Crestone Seismic Data Processing Software for ProMAX® and SeisSpace

Contact Information:

Denny O’Brien, Director of Sales and Marketing

303-909-9900

Denny@CrestoneSeismic.com

www.CrestoneSeismic.com

Velocity Analysis

Tools in this category assist the user in computing better velocities, analyzing that information and applying it in an efficient, effective manner.

Azimuthal Velocity Residual Analysis (Velresa)

This program performs azimuthal and standard velocity residual corrections to pre-stack uncorrected CDP gathers. The output from this tool is a detailed (every trace and sample) VEL or VAZ velocity table. The velocity tables can then be used to apply NMO to the seismic data. In the case of the azimuthal velocities (VAZ), we can create azimuthal anisotropy computations like Ellipticity. You can also output Vfast and Vslow to the VAZ Tool.

This tool is used to correct for vertical and horizontal velocity variations that are impossible to pick manually.

VAZ Tool

This program manipulates the azimuthal velocity data in a VAZ parameter table. The user can:

- Select velocities from a single azimuth

- Output a VEL table that is the average for all azimuthal velocities

- Perform standard deviation of the azimuthal velocities

- Compute Ellipticity and Elliptical Azimuth from the azimuthal velocities

- Output Vfast and Vslow as VEL Tables

The tool fits a least-squared ellipse algorithm to the azimuthal velocity at every trace and sample. This tool can be used to output Ellipticity which is the ratio between the fast and slow azimuthal velocities and Elliptical Azimuth which is the azimuth of the fast velocity.

CVS Velocity Analysis

The CVS velocity tool will perform interactive velocity picking of CVS panels. The input to the tool is CVS panels created with the SeisSpace Constant Velocity Stack tool. The CVS panels can be input using the Disk Data Input -> Seismic Display. Or input directly into the display using the Seismic Standalone tool.

The tool reads “N” images of CVS stacks. Then the user can move through the images quickly by rolling the mouse wheel. When the CVS stack looks the best in a certain area, the user clicks the mouse and the velocity appears on the display. The user keeps selecting new velocities over the seismic data until it is complete. The velocities are then saved to a SeisSPace VEL or PVL table.

The velocities can be picked in 3D using multiple Seismic Displays. Typically the user will load an in-line in one display and a cross line in a second display. As you roll the mouse wheel all displays will move to the same velocity panel. You can pick on either display.

Semblance Velocity Analysis w/Interactive ETA Velocity Analysis

The Semblance Velocity toll will perform interactive velocity and fourth order (ETA) picking using semblance. During analysis the user has up to four windows to help pick velocities and ETA values. A window contains the current CDP gather that can also be expanded to a super gather. A stacked section of the data is displayed in a second window, with the center traces in the stack automatically updating as you pick. The 3rd window is the semblance velocity where you pick the velocity in time and semblance. The 4th window is optionally the ETA semblance where ETA values are picked in time and semblance.

As you interactively pick velocities and ETA, you will see the CDP gather flatten and the stack improve.

Normal Moveout Correction

This program computes dynamic normal move-out corrections. There are options to control stretch mutes and perform azimuthal NMO using a VAZ table. Options to apply a 4th order (ETA) table.

This tool appears to have better stretch mutes than the ProMAX version. It also will not compute/apply the stretch on near offset traces. It will apply the VAZ azimuthal velocity tables. It can apply the ETA table from the semblance velocity analysis.

Block Move-Out (Blockem)

This program performs high resolution normal move-out with no stretch effects. Block Moveout is performed on pre-stack uncorrected CMP gathers. The output from this tool is moveout corrected CMP gathers that can then be stacked.

Time and space variant functions allow control over move-out in 3D. A mute is typically applied to the block moved-out CMP gather. The mute can be wider than a mute for regular normal moveout since no stretch occurs in the output trace.

Block Move-Out is used in shallow engineering data to give a better stack. The stretch effects are reduced in the shallow data. It also corrects for hyperbolic cross-over, when shallow unconsolidated sediments overlay a hard/fast reflector.

Paint CVS Stack

A seismic stacked section is created using Constant Velocity Stacks (CVS). The known RMS velocity is interpolated on a sample-by-sample basis for each CDP. The velocity at each window center is selected. The CVS panel nearest this velocity is chosen as the data for this window in the stacked output trace. With this tool, you can sometimes get a stack from datasets where you can't with semblance velocity analysis.

The input trace is divided into overlapping data blocks of a set time duration. The velocity is taken at the window center. The CVS trace closest to this velocity is placed into the time window. If interpolation is selected, then the stacked trace time window is interpolated from the two nearest CVS traces to the velocity.

Wavelet Stretch Compensation (Stretcomp)

Corrects for wavelet stretch on the far offset traces of NMO corrected CDP gathers. Applying NMO correction to CDP gathers causes the seismic wavelet to stretch at shallow times and large offsets. Also wavelet stretch can occur on any pre-stack dataset with increasing offset and time. Using small windows, this tool moves down in time and applies a stretch correction filter and outputs CDP gathers with stretch removed.

By Removing the stretch the processor gets more seismic events visible on the stack along with higher frequency.

Velocity Profile

Create a velocity profile that replaces the trace amplitude values at each sample with interpolated velocity values. A complete seismic dataset is output that replaces the seismic amplitudes with a corresponding velocity.

High Resolution

Tools in this category provide the user with processes to increase the frequency content of the data to assist in mapping more subtle features such as stratigraphic traps, fracture plays etc.

3D High Frequency Enhancement (Homer)

Enhance seismic data by extending the usable bandwidth of the seismic spectrum into higher frequencies. A constrained inversion is performed to compute reflection coefficients. The coefficients are then convolved with a bandwidth enhanced wavelet to create the high frequency section. The reflection coefficient series can be analyzed, interpreted, and used in additional processing.

This multi-step tool is used to get higher frequencies from the data and is typically used for stratigraphic traps and shale plays.

The Hi-Res output has achieved excellent definition of a variety of horizons which will enable the interpreter to map thin beds, faults and other stratigraphic and structural features.

Hi Res RHO Filter (RHO)

Enhance frequency bandwidth by applying a zero-phase filter of the form f^x. The high frequency information from the RHO filter is combined with the original lower frequency data to produce the broadband output.

This tool is used to get about 10 hz higher frequency out of the dataset.

Seismic Attributes

Tools in this category assist the interpreter in analyzing the data for detailed structure, faulting and fracturing.

3D Volumetric Curvature (Curvy)

Volumetric curvature attributes are valuable in mapping faults, fractures, folds, and stratigraphic features. Curvature attributes such as Most Positive, Most Negative, Mean, Gaussian, and others can be output.

This tool is used to find faults and fractures in a 3D dataset. The program was recently re-written so that it is very fast and does not have to be run between time slice/unsliced. The slicing is done inside the tool now. The user can slice thru the curvature volume with a time slice or a horizon slice.

Azimuthal Velocity Residual Analysis (Velresa)

This program performs azimuthal and standard velocity residual corrections to pre-stack uncorrected CDP gathers. The output from this tool is a detailed (every trace and sample) VEL or VAZ velocity table. The velocity tables can then be used to apply NMO to the seismic data. In the case of the azimuthal velocities (VAZ), we can create azimuthal anisotropy computations like Ellipticity. You can also output Vfast and Vslow to the VAZ Tool.

This tool is used to correct for vertical and horizontal velocity variations that are impossible to pick manually.

VAZ Tool

This program manipulates the azimuthal velocity data in a VAZ parameter table. The user can:

- Select velocities from a single azimuth

- Output a VEL table that is the average for all azimuthal velocities

- Perform standard deviation of the azimuthal velocities

- Compute Ellipticity and Elliptical Azimuth from the azimuthal velocities

- Output Vfast and Vslow as VEL Tables

The tool fits a least-squared ellipse algorithm to the azimuthal velocity at every trace and sample. This tool can be used to output Ellipticity which is the ratio between the fast and slow azimuthal velocities and Elliptical Azimuth which is the azimuth of the fast velocity.

Spectral Decomposition (Specdecomp)

This tool images and maps temporal bed thickness and geologic discontinuities over 3D seismic surveys using the discrete Fourier transform to image and map temporal bed thickness and geologic discontinuities.  It outputs such things as Frequency Spectrum. Frequency Slices can be computed in time or on horizons.

This tool is used to find stratigraphic traps like channels and reefs. It measures frequency over a defined window, either a time or a horizon, where changes in frequency can be seen. Once computed, you can pan through them.

An additional tool 'Spectral Decomposition Scaling' should be used to apply a spectral compensation factor to each frequency slice. The goal is to balance the wavelet spectrum by equalizing each frequency slice according to its average amplitude.

Spectral Decomposition Scaling

Computes a spectral compensation factor that is applied to each frequency slice. An additional method is supported that finds the local maxima after the data is scaled. The Local Maxima Frequency method will search the amplitude spectra for each sample and trace to find a local maximum in the amplitude spectrum. The output value for each sample in the output data is the frequency of the local maximum amplitude.

Noise Reduction

Tools in this category provide the user with many ways to reduce random and coherent noise to improve the overall data processing results.

3D Wavenumber Noise Suppression (Wavy)

Suppress’ noise footprints like the acquisition footprint in seismic data by 3D wavenumber filtering on seismic time slices. Several modes of operation allow you to design filters, output filtered data, subtract noise from the data, and QC the noise footprint. Special features will smooth the 3D data but leave the structural/stratigraphic geology intact.

The filters can be cascaded in the flow so that different noise footprints can be removed. Some strong noise footprints may need several of the filters cascaded to sufficiently reduce its amplitude. Special options in the tool will remove the noise footprint and smooth the data in 3D but preserve the structural/stratigraphic features.

This tool is used to remove special footprints from 3D datasets. Parts of the tool were recently re-written so that it does not have to be run between time slice/unsliced. The slicing is done inside the tool now.

Dip Azimuth Semblance and Enhancement (Dazzle)

This program builds a 3D dip and azimuth model using semblance. The model is then used to subtract noise patterns or increase the coherency of the data. The program works on pre-stack or post-stack seismic data. The dip and azimuth attributes can be used in other processing tools to compute additional attributes like 3D Volumetric Curvature.

This tool is used to increase the signal-to-noise ration in poor quality datasets. It also will increase the coherency of seismic events.

TFD Noise Removal (TFDNoise)

This program does time and frequency domain noise suppression and spectral balancing.  It attenuates noise bursts, spikes, air blasts, ocean swell, and other noise. There are several options for analyzing noise including Frequency Variant and Median Frequency. It has the option to output noise only or filtered data.

This tool appears to be much faster than the ProMAX version. It also has new filtering effects that appear to be much better than the original Amoco version. The program can output just the noise to make sure it is not attenuating data as well. You can also generate a file of noisy traces which can be input to Trace Killer based on a tolerance.

Low Frequency Kx-Ky Filter (Freqxy)

Low Frerquency Kx-Ky filtering can attenuate surface wave noise by wave number filtering in the X and Y dimension over low frequency ranges. The tool is run in the shot domain and works especially well on cross-spreads in azimuthal mode.

This tool is used to remove ground-roll and other linear noise from shots.

The tool transforms the data to the frequency Wavenumber domain. A frequency dependent mix of adjacent traces is performed in the Wavenumber domain for the specified range of frequencies. The mix is dependent on the velocity, frequency range, and X and Y wave-numbers. The filtered data is then returned to the time-space domain.

Multi-Channel Adaptive Subtraction (Subtract)

Models the noise to the data and then subtracts the noise from the data using predictive Deconvolution. You can output the signal or the modeled noise.

This tool is used to remove ground roll and other linear noise from shots.

Ring Removal (Ringy)

Removes ringing noise from seismic data using adaptive deconvolution filtering. Ringing noise can be specified by a frequency range. A noise model is built at each frequency over a user specified range of frequencies. A filter is computed using the noise model and data at each frequency and then applied.

Smoothing/Edge Filters

Reduce noise in seismic data by smoothing in 3D. Three methods used are Gaussian Blur, Sobel Edge Detection, and Symmetric Nearest Neighbor smoothing.

Smoothing the noise away will make the seismic data look more attractive and easier to interpret. Several 3D spatial filtering tools like '3D Volumetric Curvature' will benefit from smoothed data.

This tool is also used to improve the edge effects when going into coherency or Volumetric Curvature. The Sobel Edge Detection filter performs a 3D spatial gradient measurement on the data to emphasize regions that correspond to edges. The Symmetric Nearest Neighbor filter is designed to preserve edges in the data and is a very effective at noise reduction.

FXY Interpolation

Interpolate and extrapolate missing spatial locations in 3D seismic data. Also performs pre-stack FXY Deconvolution.  Inputs several types of ensembles and in-fills seismic data into empty locations to improve the spatial sampling of the data.

This tool is used to interpolate data traces before migration or DMO.

Interactive Tools

Tools in this category have a graphical interface to help view and process seismic data. These tools include seismic display, mapping, seismic input, and seismic standalone. Some of the functions performed here include CVS and semblance velocity analysis (see Velocity Analysis above), first break picking, ProMAX table picking, geometry loading, spectral analysis, SEGY input, data, attribute, and header map drawing.

Seismic Display

Interactive seismic display of data coming down the flow for 2D and 3D data on ProMAX datasets and SEGY data.

Options to change the data include window tabs that control different functions. Tabs include:

- CVS Velocity Analysis

- Semblance Velocity Analysis w/Interactive ETA Velocity Analysis

- Spectral Analysis

- Trace Header Viewing

- SeisSpace Horizon, Gate and Mute

- First Break Picking

- ProMAX Tables

- Geometry Building

- Phase and Bandpass Filter

- AGC tests

- Time Slices

- Geometry loading, viewing and editing

- Linear and normal move-out

- Image storage and more….

The user can perform data overlays and very fast image scanning.

The user can also zoom and pan over the data. Windows can be drawn on the data for zooming, spectral analysis, and trace header viewing. The display window can be saved as image files such as jpg, .png, and .pdf.

Mapper

The program will interactively map geometry information from ASCII files, the ProMAX database, or trace headers; also line ties, partial 3D extractions, printing, images, polygons, mapping attributes in 2D and 3D, and more. There are user configurable options to control the types and the way the information is displayed. The plot can be saved to several popular image file formats including .jpg, .png, and .pdf. Annotations can be drawn on the plot to bring attention to specific information. A subset of the seismic data in ProMAX can be retrieved by drawing a polygon on the map and exporting the values to the '3D Partial Extractions' tool.

When used in ASCII form, the user can load any ASCII file and interactively design what information to retrieve from the file and subsequently show on the plot. In the ProMAX method, all lines within an area are read from the ProMAX database and plotted.

Seismic Standalone

The 'Seismic Standalone' tool is a seismic display tool. But in the menu the user can specify a ProMAX dataset or SEGY dataset. The files are loaded and the user can move around the data at will. The traces are never put into the flow. This tool can be the only one in the flow.

Seismic Input Display

The 'Seismic Input/Output'  tool is an input tool. The user can specify ProMAX or SEGY datasets in the menu and move around the data at will (like the standalone tool). But the user can press the 'Process button' and the traces are moved into the flow, just like 'Disk Data Input' or 'Segy Input'.

Productivity Tools

3D Partial Extractions (Extract)

Extract partial volumes from 3D seismic data. A set of polygons define when a trace can be accepted or rejected from the dataset.

This tool is used to cutout portions of a dataset. The cutouts are requested by client to satisfy landowners and partners who don’t have privileges to all the data.

Time Slice/Un-Slice

Converts post-stack seismic data to time slices and back. It is used with Spectral Decomposition Scaling. This is used in 3D spatial filtering tools like Volumetric Curvature.

Band Limited Deconvolution

Used for spectral shaping of the wavelet before standard deconvolution.

Trace Killer

Uses trace statistics from the TFD Noise or Trace Statistic tools to kill traces. It can zero the trace or build a KIL table.

Hole Extraction

Mutes data using a HOL table. Unwanted data like noise can be removed.

Common Offset Display

Displays field data as common offsets. It can quickly see geometry problems in data while it is being shot in the field.

Vertical Stack

Vertically stacks shots from weight drop or other field devices where several shots at each point is needed to get enough energy into the ground. It can perform standard or diversity stack.

Primary/Secondary Exchange

Exchanges the primary and secondary ProMAX keys.

Rolling Supergather Create

Creates rolling supergathers in 2D or 3D. It creates the SG_CDP, SG_ILINE, and SG_XLINE header keys. This output is often used as an input into Velresa, Wavelet Stretch and Semblance Velocity Analysis

Tape Copy

Copies tapes to disk files or other tapes. It creates a key file, where, when run a second time, can remove data that you do not want copied.

ASCII XY to Z Header

Takes an ASCII file with X and Y coordinates and then writes a Z value to trace headers. The tool finds the nearest X and Y coordinate for each trace when applying the Z value header.

This program provides the user with a tool to get a Z value such as Curvature into ProMAX for display, Q.C., etc.

Trace Time Merge

Merges “N” sequential traces so that the samples are interleaved. This permits multiple datasets to be viewed with 'Time Slice” so that each sequential time slice is a different dataset. Volumetric Curvature also expects the dip and azimuth to be merged using this tool.

Energy Envelope Scaling

Boost the amplitude of low energy zones relative to high energy zones in a time a space variant manner. Improve the amplitude of events that are over-shadowed by high energy seismic events. It preserves relative amplitude.

This tool is used to improve the amplitudes of events below high energy reflectors.

Offset-Time Scaling

Boosts the amplitude of seismic data by offset and time. Two methods are employed to compute offset-time scalars: Mean Amplitude or Cross Correlation. A table of offset-time scalars is output that varies in space and time over the dataset.

This tool is used to scale the data leaving relative amplitudes and preserving AVO.

Seismic Stack   A) Normal  B) HiRes.

CDP gather with NMO  A) Normal  B) Wavelet Stretch Compensation.

- Normal seismic stack b) Seismic stack with Wavelet Stretch Compensation

3D Wavenumber Noise Reduction for removing noise footprints.

Volumetric Curvature ‘Most Positive’

Overlaying Volumetric Curvature ‘Most Positive’ onto Amplitude in a time slice using Steve’s Mapper.

Azimuthal Velocity Analysis – Ellipticity in color with the Elliptical Azimuth on top using directional arrows showing the fast velocity direction.

Dip/Azimuth Semblance using the SNR Enhancement option. Used to increase the coherency of seismic events in 3D.

Spectral Decomposition on a horizon. See the channel.

Seismic Display showing interactive menus and frequency spectrum. Can perform interactive AGC, Phase, Overlays, Bandpass Filtering, Moveout, Color Tables, High Speed Image Viewer, and more.

Seismic Display with RMS Velocity as an overlay.

Seismic Display showing some of the tools including ProMAX picking, CVS Velocity Analysis, Semblance Velocity Analysis, Refraction Picking, and Geometry Building.

Steve’s Mapper showing shots/receivers and fold map.