HTI Processing

The Crestone Seismic Software has several tools for detecting faults/fractures in shale. I have not found one method that works all the time in all cases. Many times several attributes have to be used to map the fracturing. Here are some of the methods used to map fractures:

- Velresa – Performs residual velocity analysis in azimuthal mode. The output is a VAZ table that has columns of velocities for different azimuth sectors. The velocity field is very dense, having velocity information on every trace and sample.

- Vaz Tool – Takes as input a seismic stacked dataset and a VAZ table. Will compute Ellipticity and Elliptical Azimuth. The Ellipticity is the ratio of the Vfast versus the Vslow velocities computed using a least-squares ellipse on the velocities from each azimuth sector. It will also compute average velocity and you can select of an azimuth velocity column.

- Coherency – Can be used to map fracturing. Crestone doesn’t have a Coherency tool but SeisSpace ProMAX does.

- Volumetric Curvature – You can successfully map fracturing using Volumetric Curvature. The dip and azimuth are computed prior to running Curvature. The dip and azimuth cubes are used to compute Volumetric Curvature.

If the horizons are general flat, Velresa can be used to build a high density azimuthal velocity volume (VAZ). This velocity volume can then be used in NMO to perform velocity corrections using the azimuthal velocity information.

The VAZ table can be input into the Vaz Tool to create Ellipticity and Elliptical Azimuth cubes. The Ellipticity is the ratio of Vfast versus Vslow. The Elliptical Azimuth is the direction of Vfast. Steve’s Mapper tool can plot the Ellipicity as color contours and the Elliptical Azimuth as arrows.

In the case of dip in the dataset, I have clients who first migrate the data using Azimuth sectors. Then they take the pre-stack migration cdp gathers for each azimuth and apply an un-NMO. Then you can put the data into Velresa to create the VAZ table. I am not sure how they do this, and I know they have to perform header manipulations to get src/rec coordinates back into the headers. The Velresa tool requires SRC_X, SRC_Y, REC_X, and REC_Y to compute the azimuth of each trace. The trace will also need OFFSET/AOFFSET to perform NMO during the residual calculations.

I am not sure if you can compute Vfast from the Ellipticity, since Ellipicity is the ratio of Vfast to Vslow. I will have to think about how to go about this using current SeisSpace tools. The Ellipitical Azimuth can be used as the azimuth of Vfast. I could possibly upgrade the Vaz Tool to output the computed Vfast from the least-squares ellipse. After the least-square ellipse computation, I do have the values of Vfast and Vslow, but it does not currently output these attributes.